BAUR PGK25 CABLE TEST SET

PGK 25 is a compact set for DC voltage testing with negative polarity up to 25 kV. Because of its different power supplies, this instrument can be used anywhere: a) incorporated mains supply unit and charging unit 110 - 120 / 220 - 230 / 240 V; 50 - 60 Hz b) incorporated battery 12 V c) external 12 V DC supply source possible Features: integrated timer 1-30 min ( 1 min) with automatic final disconnection and release of discharge device stepless adjustable output voltage 0 - 5 kV and 0 - 25 kV DC voltage measurement is made directly at high voltage output zero interlock of high voltage high voltage plug with control contact current measurement: decade switching in 5 ranges plotter connection for current range 0 - 200 mV (for ungrounded input) connection for external battery, 12 V